# SIMATS ENGINEERING

# Department of Computer Science & Engineering

# ASSESSMENT TOOL 1- EXPERIMENTS

| Course Code: | CSA12 | Course Title: | Computer Architecture |
| --- | --- | --- | --- |
| CO Assessed: | CO3 | Assessment: | ASSESSMENT TOOL 1-EXPERIMENTS |
| Weightage: | 45% | Total Marks: | 25 |
| CO3 Statement: | Register transfers, Arithmetic and logic operations. Multiple Bus Organization: Data transfer among multiple buses. Pipelining: Stages of pipelining, Pipeline performance, hazards (data, instruction). (BL3) |  |  |
| Student Name: | S Prithika | Reg.No.: | 192512432 |
| Department: | BE ECE | Date: | 18/8/26 |

ANNEXURE A

| Experiment | Question |
| --- | --- |
| 1 | Design and implement a Register Transfer Unit (RTU) to demonstrate data transfer between registers. Perform operations such as R2 ← R1, R3 ←R1 + R2, and R4 ← R3. Verify the contents of registers after each transfer. |
| 2 | Design and implement an Arithmetic and Logic Unit (ALU) to perform arithmetic operations such as addition and subtraction and logical operations such as AND, OR, XOR, and NOT. Display and verify the output for different input combinations. |
| 3 | Implement a Multiple Bus Organization for a CPU data path. Demonstrate the transfer of data among registers through multiple buses and perform an arithmetic operation using the ALU. Verify the register contents after each operation. |
| 4 | Design and simulate a 4-stage instruction pipeline consisting of Instruction Fetch (IF), Instruction Decode (ID), Execute (EX), and Write Back (WB). Demonstrate the execution of multiple instructions simultaneously and calculate the pipeline speedup. |
| 5 | Implement and analyze pipeline hazards using a set of dependent instructions. Demonstrate data hazards and instruction hazards, and apply appropriate techniques such as stalling, forwarding, or branch handling to resolve them. Compare the pipeline performance before and after hazard resolution. |

Note: use Logisim

# 1. Register Transfer Unit (RTU)

Aim

To design and implement a Register Transfer Unit to demonstrate data transfer between registers and verify the register contents after each operation.

Operations

  - R2 ← R1

  - R3 ← R1 + R2

  - R4 ← R3 Verilog Code module RTU;

reg [7:0] R1, R2, R3, R4;

initial begin

// Initial values R1 = 8'd10; R2 = 8'd0;

R3 = 8'd0; R4 = 8'd0;

$display("Initial: R1=%d R2=%d R3=%d R4=%d", R1,R2,R3,R4);

// R2 <- R1 R2 = R1;

$display("After R2 <- R1: R1=%d R2=%d R3=%d R4=%d", R1,R2,R3,R4);

// R3 <- R1 + R2 R3 = R1 + R2;

$display("After R3 <- R1 + R2: R1=%d R2=%d R3=%d R4=%d", R1,R2,R3,R4);

// R4 <- R3 R4 = R3;

$display("After R4 <- R3: R1=%d R2=%d R3=%d R4=%d", R1,R2,R3,R4);

$finish; end

endmodule Expected Output

| Operation | R1 | R2 | R3 | R4 |
| --- | --- | --- | --- | --- |
| Initial | 10 | 0 | 0 | 0 |
| R2 ← R1 | 10 | 10 | 0 | 0 |

Operation	R1	R2	R3	R4

| R3 ← R1 + R2 | 10 | 10 | 20 | 0 |
| --- | --- | --- | --- | --- |
| R4 ← R3 | 10 | 10 | 20 | 20 |

Result

The Register Transfer Unit successfully performs the required register transfers and arithmetic operation. The final contents are:

R1 = 10, R2 = 10, R3 = 20, R4 = 20.

# Arithmetic and Logic Unit (ALU)

Aim

To design and implement an ALU that performs arithmetic operations such as addition and subtraction and logical operations such as AND, OR, XOR, and NOT.

Operations

| Opcode | Operation |
| --- | --- |
| 000 | Addition |
| 001 | Subtraction |
| 010 | AND |
| 011 | OR |
| 100 | XOR |
| 101 | NOT A |

Verilog Code module ALU;

reg [7:0] A, B;

reg [2:0] OP;

reg [7:0] Y;

always @(*) begin case(OP)

3'b000: Y = A + B;

3'b001: Y = A - B;

3'b010: Y = A & B;

3'b011: Y = A | B;

3'b100: Y = A ^ B;

3'b101: Y = ~A;

default: Y = 8'b00000000; endcase

end

initial begin

A = 8'b00001111;	// 15

B = 8'b00000011;	// 3

OP = 3'b000; #10;

$display("ADD : %d", Y);

OP = 3'b001; #10;

$display("SUB : %d", Y);

OP = 3'b010; #10;

$display("AND : %d", Y);

OP = 3'b011; #10;

$display("OR : %d", Y);

OP = 3'b100; #10;

$display("XOR : %d", Y);

OP = 3'b101; #10;

$display("NOT : %d", Y);

$finish; end

endmodule Expected Output For:

o A = 15 = 00001111

o B = 3 = 00000011

| Operation | Result |
| --- | --- |
| A + B | 18 |
| A - B | 12 |
| A AND B | 3 |
| A OR B | 15 |
| A XOR B | 12 |
| NOT A | 240 |

NOT A = 11110000 = 240 for an 8-bit ALU.

Result

The ALU successfully performs addition, subtraction, AND, OR, XOR, and NOT

operations for the given input combinations.

# Multiple Bus Organization

Aim

To implement a multiple-bus organization for a CPU datapath, demonstrate data transfer between registers through multiple buses, and perform an arithmetic operation using the ALU.

Basic Organization

A simple three-bus datapath consists of:

  - Bus A → carries first ALU operand

  - Bus B → carries second ALU operand

  - Bus C → carries ALU result back to a register Example operation:

R3 ← R1 + R2

The operation occurs as:

R1 ──> Bus A ──┐

│

▼

ALU ───> Bus C ───> R3

▲

│

R2 ──> Bus B ──┘ Verilog Code

module Multiple_Bus;

reg [7:0] R1, R2, R3, R4;

reg [7:0] BusA, BusB, BusC; initial begin

// Load registers R1 = 8'd15;

R2 = 8'd5;

R3 = 8'd0; R4 = 8'd0;

$display("Initial: R1=%d R2=%d R3=%d R4=%d", R1,R2,R3,R4);

// R1 -> Bus A BusA = R1;

// R2 -> Bus B BusB = R2;

// ALU addition

BusC = BusA + BusB;

// Bus C -> R3 R3 = BusC;

$display("After R3 <- R1 + R2:");

$display("R1=%d R2=%d R3=%d R4=%d", R1,R2,R3,R4);

// R3 -> R4 BusA = R3; R4 = BusA;

$display("After R4 <- R3:");

$display("R1=%d R2=%d R3=%d R4=%d", R1,R2,R3,R4);

$finish; end

endmodule Expected Output

| Operation | R1 | R2 | R3 | R4 |
| --- | --- | --- | --- | --- |
| Initial | 15 | 5 | 0 | 0 |
| R3 ← R1 + R2 | 15 | 5 | 20 | 0 |
| R4 ← R3 | 15 | 5 | 20 | 20 |

Result

The multiple-bus CPU datapath successfully transfers data through Bus A, Bus B, and Bus

C. The ALU performs the addition and stores the result in R3. Final values: R1 = 15, R2 = 5, R3 = 20, R4 = 20.

# Four-Stage Instruction Pipeline

Aim

To design and simulate a four-stage instruction pipeline consisting of:

- IF – Instruction Fetch

- ID – Instruction Decode

- EX – Execute

- WB – Write Back

and calculate the pipeline speedup. Pipeline Stages

IF → ID → EX → WB

Consider four instructions:

I1 I2 I3 I4

Pipeline Timing

| Cycle | I1 | I2 | I3 | I4 |
| --- | --- | --- | --- | --- |
| 1 | IF |  |  |  |
| 2 | ID | IF |  |  |
| 3 | EX | ID | IF |  |
| 4 | WB | EX | ID | IF |
| 5 |  | WB | EX | ID |
| 6 |  |  | WB | EX |
| 7 |  |  |  | WB |

Therefore, 4 instructions require 7 cycles using the pipeline. Non-Pipelined Execution

Without pipelining, each instruction requires 4 stages: 4 instructions × 4 stages = 16 cycles

Pipelined Execution

For n instructions and k stages:

𝑇𝑝𝑖𝑝𝑒𝑙𝑖𝑛𝑒 = 𝑘 + (𝑛 − 1)

For k = 4 and n = 4: Speedup

𝑇	= 4 + (4 − 1) = 7 𝑐𝑦𝑐𝑙𝑒𝑠

𝑝𝑖𝑝𝑒𝑙𝑖𝑛𝑒

𝑇

𝑇

𝑆𝑝𝑒𝑒𝑑𝑢𝑝 =   𝑛𝑜𝑛−𝑝𝑖𝑝𝑒𝑙𝑖𝑛𝑒

𝑝𝑖𝑝𝑒𝑙𝑖𝑛𝑒

7

𝑆𝑝𝑒𝑒𝑑𝑢𝑝 =  16

𝑆𝑝𝑒𝑒𝑑𝑢𝑝 = 2. 286

So the pipeline gives approximately 2.29× speedup for four instructions.

Simple Verilog Simulation module Pipeline;

integer cycle; initial begin

$display("Cycle\tIF\tID\tEX\tWB");

for(cycle = 1; cycle <= 7; cycle = cycle + 1) begin case(cycle)

1: $display("1\tI1\t-\t-\t-");

2: $display("2\tI2\tI1\t-\t-");

3: $display("3\tI3\tI2\tI1\t-");

4: $display("4\tI4\tI3\tI2\tI1");

5: $display("5\t-\tI4\tI3\tI2");

6: $display("6\t-\t-\tI4\tI3");

7: $display("7\t-\t-\t-\tI4"); endcase

end

$finish; end

endmodule Result

The four-stage pipeline successfully executes multiple instructions simultaneously. For four instructions, execution time is reduced from 16 cycles to 7 cycles, giving a speedup of approximately 2.29×.

# Pipeline Hazards and Hazard Resolution

Aim

To implement and analyze pipeline hazards using dependent instructions and demonstrate techniques such as stalling, forwarding, and branch handling.

Types of Hazards

- Data Hazard Consider:

I1: R1 ← R2 + R3 I2: R4 ← R1 + R5

I2 requires the value of R1 produced by I1.

This creates a RAW (Read After Write) data hazard. Without Forwarding

The pipeline may have to stall:

| Cycle | I1 | I2 |
| --- | --- | --- |
| 1 | IF |  |
| 2 | ID | IF |
| 3 | EX | ID |
| 4 | MEM/EX | STALL |
| 5 | WB | STALL |
| 6 |  | EX |
| 7 |  | WB |

The stall increases execution time.

- Forwarding Technique

Instead of waiting for the value to be written back to the register, the ALU result from I1 can be directly forwarded to the ALU input of I2.

I1 ALU Result

│

▼ Forwarding Unit

│

▼

I2 ALU Input

This reduces or eliminates unnecessary stalls. Example

I1: ADD R1, R2, R3 I2: SUB R4, R1, R5

The result of I1 is forwarded directly to I2.

- Instruction/Control Hazard

A branch instruction creates a control hazard because the processor does not immediately know which instruction should be fetched next.

Example:

I1: BEQ R1, R2, TARGET I2: ADD R3, R4, R5

I3: SUB R6, R7, R8

If the branch is taken, instructions already fetched after the branch may be incorrect. Branch Handling

A common solution is to:

- Stall until the branch decision is known, or

- Flush incorrectly fetched instructions, or

- Use branch prediction.

For a simple pipeline, stalling and flushing can be used.

Pipeline Performance Comparison

Assume 5 instructions require 9 cycles without hazards. If two stall cycles are introduced:

𝑇	= 9 + 2 = 11 𝑐𝑦𝑐𝑙𝑒𝑠

ℎ𝑎𝑧𝑎𝑟𝑑

With forwarding, the two unnecessary stalls can be removed:

𝑇	= 9 𝑐𝑦𝑐𝑙𝑒𝑠

𝑓𝑜𝑟𝑤𝑎𝑟𝑑𝑖𝑛𝑔

Comparison

| Method | Cycles | Performance |
| --- | --- | --- |
| Normal pipeline | 9 | Best |
| With stalls | 11 | Lower |
| With forwarding | 9 | Improved |
| With branch flush | Depends on branch | Reduced |

Simple Verilog Example module Pipeline_Hazard;

reg [7:0] R1, R2, R3, R4, R5;

initial begin

R2 = 8'd10; R3 = 8'd20; R5 = 8'd5;

// I1: R1 = R2 + R3 R1 = R2 + R3;

// I2: R4 = R1 + R5

// Forwarded value of R1 is used R4 = R1 + R5;

$display("After I1: R1 = %d", R1);

$display("After I2: R4 = %d", R4);

$finish; end

endmodule Expected Output After I1: R1 = 30 After I2: R4 = 35 Result

The pipeline hazards were successfully identified and resolved. Data hazards were handled using forwarding, while instruction/control hazards can be handled using stalling, flushing, or branch prediction. Forwarding improves performance by reducing unnecessary pipeline stalls.

Final Viva/Record Summary

| Experiment | Main Concept | Final Result |
| --- | --- | --- |
| 1. RTU | Register transfer | R2=R1, R3=R1+R2, R4=R3 verified |
| 2. ALU | Arithmetic & logic | ADD, SUB, AND, OR, XOR, NOT verified |
| 3. Multiple Bus | CPU datapath | Data transferred through multiple buses and ALU operation verified |
| 4. Pipeline | IF-ID-EX-W B | 4 instructions completed in 7 cycles; speedup = 2.29× |
| 5. Hazards | Stalling & forwarding | Data/control hazards identified and resolved |

Code of Conduct:

I S.PRITHIKA(192512432)certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student:

MARKS ALLOCATION

|  | Problem Understanding (20%) | Method Selection (20%) | Solution Process (25%) | Accuracy of Calculation (20%) | Interpretation of Results (15%) |
| --- | --- | --- | --- | --- | --- |
| Q1 |  |  |  |  |  |
| Q2 |  |  |  |  |  |
| Q3 |  |  |  |  |  |
| Q4 |  |  |  |  |  |
| Q5 |  |  |  |  |  |

RUBRICS FOR CALCULATION:

| Criteria | Weightage | Level 4 – Exemplary | Level 3 – Proficient | Level 2 – Developing | Level 1 – Beginnin g |
| --- | --- | --- | --- | --- | --- |
| Problem Understanding | 20% | Clearly interprets problem, identifies all parameters and requirements accurately | Understands problem with minor gaps | Partial understanding; misses key elements | Misinterprets or unable to understand problem |

| Method Selection | 20% | Selects most appropriate method/formula with strong justification | Selects correct method with minor errors | Inappropriate or partially correct method | Unable to select method |
| --- | --- | --- | --- | --- | --- |
| Solution Process | 25% | Logical, systematic steps with correct approach throughout | Mostly correct steps with minor mistakes | Disorganized steps; multiple errors | No clear process |
| Accuracy of Calculation | 20% | All calculations accurate; correct final answer | Minor calculation errors | Frequent errors; incorrect final answer | No valid calculations |
| Interpretation of Results | 15% | Clearly interprets results with units and engineering relevance | Basic interpretation with minor omissions | Limited or unclear interpretation | No interpretation |