SIMATS ENGINEERING

Department of Computer Science & Engineering

ASSESSMENT TOOL 1- ANALYTICAL PROBLEM SOLVING

| Course Code: | CSA12 | Course Title: | Computer Architecture |
| --- | --- | --- | --- |
| CO Assessed: | CO3 | Assessment: | ASSESSMENT TOOL 2-MCQ |
| Weightage: | 20% | Total Marks: | 50 |
| CO3 Statement: | Superscalar Processing: Out-of-order execution, Speculative execution, Dispatch and execution units. Advanced Features: Hyper-threading and simultaneous multithreading (SMT). (BL3) |  |  |
| Student Name: | K Kaviya | Reg.No.: | 192512077 |
| Department: | BE ECE | Date: | 18-07-2026 |

ANNEXURE A

MCQ Questions

- A 4-issue superscalar processor can ideally issue how many instructions per clock cycle?

A) 1

B) 2

C) 4

D) 8

- A processor has four execution units, but only two independent instructions are

available in a cycle. What is the maximum number of instructions that can be executed in that cycle?

A) 1

B) 2

C) 4

D) 8

- Which situation most directly limits the performance of a superscalar processor?

  - High instruction-level parallelism

  - Independent instructions

  - Strong data dependencies

  - Multiple execution units

- Consider the instructions: I1: R1 ← R2 + R3 and I2: R4 ← R5 + R6. If sufficient execution units are available, what can happen?

  - I2 must wait for I1

  - Both can execute concurrently

  - I1 must wait for I2

  - Neither can execute

- A 4-wide superscalar processor executes an average of 2 instructions per cycle. What is its effective IPC?

A) 1

B) 2

C) 4

D) 8

- If a processor's issue width is increased from 2 to 4 but the available instruction-level parallelism remains low, what is the likely result?

  - Performance always doubles

  - Performance may improve only slightly

  - Performance becomes zero

  - Clock frequency must double

- Which mechanism dynamically identifies instructions that are ready for execution?

  - Scheduler

  - Hard disk controller

  - DMA controller

  - Memory decoder

- Why are multiple execution units important in superscalar processors?

  - They allow independent operations to proceed simultaneously

  - They eliminate the need for instructions

  - They replace main memory

  - They reduce program size

- Consider: I1: R1 ← R2 + R3 and I2: R4 ← R1 × R5. Why can I2 not execute before I1 produces R1?

  - Control dependency

  - RAW dependency

  - WAW dependency

  - Structural dependency

- Which technique allows a processor to exploit instructions that are independent even when earlier instructions are delayed?

  - Out-of-order execution

  - Sequential execution

  - Memory mapping

  - Paging

- Instruction I1 is waiting for memory data, while I2 is independent and ready. What can an out-of-order processor do?

  - Stall I2 until I1 finishes

  - Execute I2 while I1 waits

  - Delete I1

  - Restart the processor

- What is the major advantage of out-of-order execution?

  - It keeps execution units busy

  - It eliminates all memory accesses

  - It reduces the number of registers

  - It eliminates branch instructions

- Consider the sequence: I1: R1 ← R2 + R3, I2: R4 ← R1 + R5, I3: R6 ← R7 + R8. If I1 is delayed, which instruction can potentially execute first?

  - I2 only

  - I3 only

  - I1 only

  - I1 and I2 only

- Why is register renaming used in an out-of-order processor?

  - To eliminate false register dependencies

  - To increase instruction length

  - To reduce cache size

  - To remove the ALU

- Two instructions write to the same architectural register but have no true data dependency. Which hazard can occur?

  - RAW

  - WAR

  - WAW

  - Cache hazard

- Register renaming primarily converts architectural registers into:

  - Physical registers

  - Cache blocks

  - Memory pages

  - Instruction codes

- Which dependency represents a true data dependency?

  - RAW

  - WAR

  - WAW

  - None

- If instruction I2 reads a register before instruction I1 writes a new value to that register, the dependency is:

  - RAW

  - WAR

  - WAW

  - Structural

- Which structure helps an out-of-order processor preserve the original program order when committing results?

  - Reorder Buffer

  - ALU

  - Branch Predictor

  - Cache Controller

- Why should results generally be committed in program order?

  - To maintain precise architectural state

  - To increase instruction length

  - To reduce register count

  - To disable speculation

- An instruction has completed execution but cannot yet update the architectural state because an older instruction has not completed. Which mechanism can handle this situation?

  - Reorder Buffer

  - RAM refresh

  - DMA

  - I/O controller

- Which situation is most suitable for out-of-order execution?

  - Independent instructions with different execution delays

  - A sequence where every instruction depends on the previous one

  - A processor with no execution units

  - A program containing no instructions

- Why does a processor use speculative execution?

  - To continue useful execution before branch outcomes are known

  - To eliminate registers

  - To reduce memory capacity

  - To stop pipeline execution

- A branch predictor predicts that a conditional branch will be taken. What does the processor typically do?

  - Fetch and possibly execute instructions from the predicted path

  - Stop fetching instructions permanently

  - Delete the branch instruction

  - Reset the processor

- What happens when speculative execution follows an incorrect branch prediction?

  - Incorrect speculative work is discarded

  - All previous programs are deleted

  - The processor permanently stops

  - The cache is destroyed

- A pipeline has a branch misprediction penalty of 5 cycles. If 10 branch mispredictions occur, how many cycles are lost, assuming no overlap?

  - 5 cycles

  - 10 cycles

  - 50 cycles

  - 100 cycles

- Which factor has the greatest impact on the effectiveness of speculative execution?

  - Branch prediction accuracy

  - Keyboard speed

  - Monitor resolution

  - Disk capacity

- If branch prediction accuracy decreases significantly, what is the likely effect?

  - More pipeline flushes and reduced performance

  - Increased memory capacity

  - Elimination of all hazards

  - Increased instruction length

- Which prediction would generally be most beneficial for a branch executed 100 times and taken 95 times?

  - Predict taken

  - Predict not taken

  - Predict randomly

  - Never predict

- Speculative instructions must not permanently change the architectural state until:

  - Their correctness is confirmed

  - They enter the cache

  - The program terminates

  - The clock stops

- Which mechanism helps discard the results of incorrectly speculated instructions?

  - Recovery mechanism

  - Cache replacement

  - DMA controller

  - Memory decoder

- What is the major performance benefit of accurate branch prediction?

  - Reduced control-flow stalls

  - Reduced RAM size

  - Reduced instruction count

  - Elimination of registers

- An instruction requiring floating-point addition is dispatched to which type of execution unit?

  - Floating-Point Unit

  - Integer ALU

  - Branch Unit

  - Load/Store Unit

- A load instruction generally requires which execution resource?

  - Load/Store Unit

  - Floating-Point Unit

  - Branch Predictor

  - Integer Register only

- If all integer ALUs are busy but a floating-point unit is free, what is the likely result for a ready integer instruction?

  - It waits for an appropriate integer execution unit

  - It executes on the floating-point unit automatically

  - It is deleted

  - It becomes a branch instruction

- What is the primary role of the dispatch stage?

  - Send instructions toward appropriate execution resources

  - Store the operating system

  - Perform disk operations

  - Generate program source code

- A processor has two integer ALUs and one floating-point unit. Which instruction mix can best utilize all three simultaneously?

  - Two integer operations and one floating-point operation

  - Three floating-point operations

  - Three branch instructions only

  - One integer operation only

- If execution units are frequently idle because instructions depend on previous results, the main limitation is:

  - Lack of instruction-level parallelism

  - Excessive memory capacity

  - Excessive register count

  - High display resolution

- A physical CPU core supports two hardware threads using SMT. How many logical processors can the operating system typically see from that core?

A) 1

B) 2

C) 4

D) 8

- Two threads execute on the same SMT core. Which statement is correct?

  - They can share the core's execution resources

  - Each thread receives a completely separate physical core

  - They cannot execute simultaneously

  - One thread must always remain inactive

- SMT is most useful when:

  - One thread leaves some processor resources underutilized

  - The processor has no execution units

  - Only one instruction exists

  - The system has no memory

- Which statement best distinguishes SMT from multicore processing?

  - SMT allows multiple threads to share one physical core

  - SMT requires one physical core per thread

  - Multicore uses only one execution stream

  - Multicore eliminates parallelism

- A single-threaded workload completely occupies all execution resources of a core. What is the likely benefit of enabling SMT for that workload?

  - Limited performance improvement

  - Guaranteed 2× performance

  - Guaranteed 4× performance

  - Performance becomes zero

- Two threads on the same SMT core compete heavily for the same execution resources. What is the likely result?

  - Performance may not scale linearly

  - Performance always doubles

  - Both threads stop permanently

  - Memory capacity doubles

- Which resources may be shared by multiple SMT threads?

  - Execution units and caches

  - Separate physical cores only

  - Separate processors only

  - Independent motherboards

- Why does SMT generally provide less performance improvement than adding a completely independent physical core?

  - Threads share hardware resources within the same core

  - SMT uses no registers

  - SMT disables pipelining

  - SMT eliminates execution units

- A processor has 4 physical cores and each core supports 2-way SMT. How many logical processors can the operating system typically detect?

  - 4

  - 6

  - 8

  - 16

- Which workload is most likely to benefit from SMT?

  - Multiple threads with complementary resource demands

  - A single instruction with no parallelism

  - A program that never executes

  - A workload with completely unavailable memory

- Which combination best represents techniques used to exploit instruction and thread-level parallelism?

  - Superscalar execution and SMT

  - Paging and segmentation

  - DMA and interrupt handling

  - Cache replacement and disk formatting

- A processor combines superscalar execution, out-of-order execution, speculative execution, and SMT. What is the primary overall goal?

  - Maximize utilization of processor resources and improve throughput

  - Reduce the number of instructions in a program to zero

  - Eliminate the need for main memory

  - Replace the operating system with hardware

1.

A 4-issue superscalar processor can ideally issue four instructions in one clock cycle.
This requires enough independent instructions and available execution resources.

2.

Only two independent instructions are available in the cycle, so only two can execute.
The number of execution units does not matter if there are not enough ready instructions.

3.

Strong data dependencies prevent instructions from executing independently.
This reduces instruction-level parallelism and lowers processor performance.

4.

The two instructions use different registers and do not depend on each other.
Therefore, they can execute concurrently when sufficient execution units are available.

5.

IPC represents the average number of instructions executed per clock cycle.
An average of two instructions per cycle gives an effective IPC of 2.

6.

Increasing issue width does not help much when available instruction-level parallelism is low.
Therefore, performance may improve only slightly.

7.

The scheduler identifies instructions whose operands are ready for execution.
It then selects suitable instructions and sends them to execution units.

8.

Multiple execution units allow independent instructions to execute simultaneously.
This increases parallelism and improves processor throughput.

9.

The second instruction requires the result produced by the first instruction.
Therefore, it must wait until the required register value is available.

10.

Out-of-order execution allows independent ready instructions to execute before delayed instructions.
This prevents unnecessary waiting and keeps processor resources busy.

11.

The second instruction is independent and ready while the first waits for memory data.
Therefore, the processor can execute the second instruction immediately.

12.

Out-of-order execution allows ready instructions to execute while others are stalled.
This keeps execution units busy and improves overall processor utilization.

13.

The second instruction depends on the delayed first instruction and must wait.
The third instruction is independent, so it can execute first.

14.

Register renaming removes false dependencies between instructions using the same architectural registers.
It mainly helps eliminate unnecessary WAR and WAW hazards.

15.

When two instructions write to the same register without a true dependency, their writes may conflict.
This situation is known as a Write After Write hazard.

16.

Register renaming maps architectural registers to physical registers.
This provides additional registers for efficient out-of-order execution.

17.

A true data dependency occurs when a later instruction needs a result produced by an earlier instruction.
This dependency is represented by Read After Write.

18.

The earlier instruction reads a register before the later instruction writes a new value.
This creates a Write After Read dependency.

19.

The Reorder Buffer keeps track of instructions that have been issued and executed.
It helps ensure that results are committed in the original program order.

20.

Committing results in program order maintains a correct and precise architectural state.
It also helps the processor handle exceptions and mispredictions correctly.

21.

A completed instruction can remain in the Reorder Buffer until older instructions finish.
It can then update the architectural state in the correct order.

22.

Out-of-order execution is most useful when independent instructions are available.
They can execute while other instructions with longer delays are waiting.

23.

Speculative execution allows the processor to continue working before a branch result is known.
This reduces waiting and keeps the instruction pipeline active.

24.

When a branch is predicted as taken, the processor follows the predicted path.
It fetches and may execute instructions from that path before confirmation.

25.

If the prediction is wrong, instructions from the incorrect path are no longer useful.
Their speculative work is discarded and execution continues from the correct path.

26.

Each branch misprediction causes a five-cycle penalty.
For ten mispredictions, the total lost time is 50 cycles.

27.

Branch prediction accuracy strongly affects speculative execution performance.
Accurate predictions reduce wrong-path execution and pipeline flushes.

28.

Lower branch prediction accuracy causes more incorrect predictions.
This results in more pipeline flushes and reduced processor performance.

29.

Since the branch is taken 95 out of 100 times, predicting it as taken is more effective.
This prediction would achieve approximately 95% accuracy.

30.

Speculative instructions should not permanently modify the architectural state immediately.
Their results become permanent only after their correctness is confirmed.

31.

A recovery mechanism handles the processor state after an incorrect speculation.
It discards incorrect results and restores execution to the correct path.

32.

Accurate branch prediction allows the processor to continue fetching useful instructions.
This reduces control-flow stalls and improves pipeline performance.

33.

Floating-point addition requires specialized floating-point arithmetic hardware.
Therefore, the operation is handled by the Floating-Point Unit.

34.

A load instruction transfers data from memory into the processor.
This operation is handled by the Load/Store Unit.

35.

An integer instruction normally requires an available integer execution unit.
It must wait if all suitable integer ALUs are busy.

36.

The dispatch stage sends instructions toward their appropriate execution resources.
This ensures that different instruction types reach suitable processing units.

37.

Two integer operations can use the two integer ALUs at the same time.
A floating-point operation can simultaneously use the available floating-point unit.

38.

Dependencies between instructions reduce the number of operations that can execute independently.
This lack of instruction-level parallelism leaves execution units idle.

39.

Two-way SMT allows two hardware threads to operate on one physical core.
Therefore, the operating system typically detects two logical processors.

40.

SMT allows multiple threads to use the resources of the same physical core.
The threads can execute concurrently when suitable resources are available.

41.

SMT is useful when a thread does not fully utilize all processor resources.
Another thread can use some of the otherwise idle resources.

42.

SMT allows multiple threads to share a single physical processor core.
Multicore processing instead provides separate physical cores for execution.

43.

If one thread already completely occupies the core's resources, little capacity remains.
Therefore, enabling SMT provides only limited performance improvement.

44.

Two SMT threads may compete for the same execution resources.
Because of this competition, performance may not increase linearly.

45.

SMT threads can share hardware resources such as execution units and caches.
These shared resources allow multiple threads to operate on one physical core.

46.

SMT threads share the hardware resources of the same physical core.
A separate physical core provides additional dedicated execution resources.

47.

Each physical core supports two hardware threads through two-way SMT.
Four cores therefore provide a total of eight logical processors.

48.

SMT benefits workloads where different threads have complementary resource requirements.
One thread can use resources that another thread leaves underutilized.

49.

Superscalar execution exploits parallelism among instructions within a thread.
SMT exploits parallelism by allowing multiple threads to share a core.

50.

Combining these techniques allows more instructions and threads to execute efficiently in parallel.
The overall goal is to maximize processor resource utilization and improve throughput.

Code of Conduct:

I K KAVIYA (192512077)certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student:

K KAVIYA

MARKS ALLOCATION

|  | Problem Understanding (20%) | Method Selection (20%) | Solution Process (25%) | Accuracy of Calculation (20%) | Interpretation of Results (15%) |
| --- | --- | --- | --- | --- | --- |
| Q1 |  |  |  |  |  |
| Q2 |  |  |  |  |  |
| Q3 |  |  |  |  |  |
| Q4 |  |  |  |  |  |
| Q5 |  |  |  |  |  |

RUBRICS FOR CALCULATION:

| Criteria | Weightage | Level 4 – Exemplary | Level 3 – Proficient | Level 2 – Developing | Level 1 – Beginning |
| --- | --- | --- | --- | --- | --- |
| Problem Understanding | 20% | Clearly interprets problem, identifies all parameters and requirements accurately | Understands problem with minor gaps | Partial understanding; misses key elements | Misinterprets or unable to understand problem |
| Method Selection | 20% | Selects most appropriate method/formula with strong justification | Selects correct method with minor errors | Inappropriate or partially correct method | Unable to select method |
| Solution Process | 25% | Logical, systematic steps with correct approach throughout | Mostly correct steps with minor mistakes | Disorganized steps; multiple errors | No clear process |
| Accuracy of Calculation | 20% | All calculations accurate; correct final answer | Minor calculation errors | Frequent errors; incorrect final answer | No valid calculations |
| Interpretation of Results | 15% | Clearly interprets results with units and engineering relevance | Basic interpretation with minor omissions | Limited or unclear interpretation | No interpretation |