SIMATS ENGINEERING

Department of Computer Science & Engineering

ASSESSMENT TOOL 1- ANALYTICAL PROBLEM SOLVING

| Course Code: | CSA12 | Course Title: | Computer Architecture |
| --- | --- | --- | --- |
| CO Assessed: | CO2 | Assessment: | ASSESSMENT TOOL 1- ANALYTICAL PROBLEM SOLVING |
| Weightage: | 45% | Total Marks: | 25 |
| CO2 Statement: | Apply integer and floating-point arithmetic algorithms including Booth's multiplication, restoring/non-restoring division, and IEEE 754 standard operations to solve fixed-point and floating-point computational problems. (BL3) |  |  |
| Student Name: | Kaviya K | Reg.No.: | 192512077 |
| Department: | BE ECE | Date: |  |

ANNEXURE A

1. A Booth's multiplication implementation produces an incorrect result when multiplying            -13 *9. Analyze the execution steps to identify the incorrect addition, subtraction, or arithmetic shift operation responsible for the error. Correct the algorithm and suggest an optimization that minimizes the number of execution cycles.

- Error Source: The incorrect result arises from performing a wrong subtraction/addition step when the Booths algorithm encounters the transition 10 (subtract multiplicand) or 01 (add multiplicand). In the case of -13 × 9, the subtraction step was mishandled.

- Correction: Ensure that when the pair (Qn, Qn+1) = 10, the algorithm subtracts the multiplicand correctly, and when (Qn, Qn+1) = 01, it adds the multiplicand. The arithmetic right shift must propagate the sign bit.

- Optimization: Use modified Booth36s algorithm (radix-4), which reduces the number of cycles by handling two bits at a time, cutting execution steps nearly in half.

2. A processor executing the restoring division algorithm produces an incorrect quotient for the operation 29 /5. Examine the sequence of iterations to determine the faulty restore step, correct the error, and recommend an optimization to improve the algorithm's efficiency.

- Error Source: The faulty step occurs when the partial remainder becomes negative, and the algorithm incorrectly restores it. Instead of restoring, the quotient bit was set incorrectly.

- Correction: After subtraction, if the remainder is negative, restore by adding the divisor back and set the quotient bit to 0. If positive, keep the remainder and set the quotient bit to 1.

- Optimization: Use non-restoring division or SRT division, which avoids unnecessary restore steps, improving efficiency.

3. During the execution of the non-restoring division algorithm, incorrect quotient bits are generated after the fourth iteration. Investigate the source of the logic error, modify the algorithm to eliminate the issue, and verify the corrected quotient and remainder.

- Error Source: The logic error lies in misinterpreting the sign of the partial remainder. The algorithm incorrectly assigns quotient bits when the remainder switches sign.

- Correction: Modify the rule:

  - If remainder ≥ 0 → subtract divisor, quotient bit = 1.

  - If remainder < 0 → add divisor, quotient bit = 0. At the end, if remainder < 0, add divisor once more to finalize.

- Verification: For 29 ÷ 5, corrected quotient = 5, remainder = 4.

- Optimization: Use carry-save adders or pipeline the division steps to reduce cycle count.

4. A Booth's multiplication unit incorrectly performs logical right shifts instead of arithmetic right shifts, leading to erroneous results for signed multiplication. Identify the cause of the error, explain why arithmetic right shifting is required, and demonstrate how it ensures correct signed multiplication.

- Error Source: Logical right shift was used, which fills the leftmost bit with 0. This destroys the sign information for negative numbers.

- Why Arithmetic Shift is Required: Arithmetic right shift replicates the sign bit (MSB), preserving two’s complement representation during shifts. Without it, signed multiplication produces incorrect results.

- Correction: Replace logical right shift with arithmetic right shift in the hardware unit.

- Demonstration: For negative numbers like -13, arithmetic shift ensures the sign bit remains 1, maintaining correctness in partial product accumulation.

5. A floating-point addition program based on the IEEE 754 single-precision format produces an incorrect result because the operands' exponents are not aligned before addition. Identify the implementation error, explain the correct exponent alignment procedure, and propose an optimized approach to improve both accuracy and execution efficiency.

  Error Source: Operands’ exponents were not aligned before addition, leading to incorrect mantissa addition.

  Correct Procedure:

- Compare exponents.

- Shift the mantissa of the smaller exponent right until exponents match.

- Add mantissas.

- Normalize the result (adjust exponent and mantissa).

- Round according to IEEE rules.

  Optimization: Use leading zero detectors and fast shifters to align exponents quickly, reducing latency. Also, employ fused multiply-add (FMA) units for efficiency in chained operations.

Code of Conduct:

I _Kaviya K (192512077)___certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student:

Kaviya K

MARKS ALLOCATION

|  | Problem Understanding (20%) | Method Selection (20%) | Solution Process (25%) | Accuracy of Calculation (20%) | Interpretation of Results (15%) |
| --- | --- | --- | --- | --- | --- |
| Q1 |  |  |  |  |  |
| Q2 |  |  |  |  |  |
| Q3 |  |  |  |  |  |
| Q4 |  |  |  |  |  |
| Q5 |  |  |  |  |  |

RUBRICS FOR CALCULATION:

| Criteria | Weightage | Level 4 – Exemplary | Level 3 – Proficient | Level 2 – Developing | Level 1 – Beginning |
| --- | --- | --- | --- | --- | --- |
| Problem Understanding | 20% | Clearly interprets problem, identifies all parameters and requirements accurately | Understands problem with minor gaps | Partial understanding; misses key elements | Misinterprets or unable to understand problem |
| Method Selection | 20% | Selects most appropriate method/formula with strong justification | Selects correct method with minor errors | Inappropriate or partially correct method | Unable to select method |
| Solution Process | 25% | Logical, systematic steps with correct approach throughout | Mostly correct steps with minor mistakes | Disorganized steps; multiple errors | No clear process |
| Accuracy of Calculation | 20% | All calculations accurate; correct final answer | Minor calculation errors | Frequent errors; incorrect final answer | No valid calculations |
| Interpretation of Results | 15% | Clearly interprets results with units and engineering relevance | Basic interpretation with minor omissions | Limited or unclear interpretation | No interpretation |